# 5G - Fundamentals of Electronic Communication

# 1 of 14 Welcome

Welcome to this session on the fundamentals of electronic communication.

In this session we will:

- Introduce the basic concept of wave theory

- Explore the electromagnetic spectrum

# 2 of 14 Definitions

Sine waves are used in math and physics to represent waves. See below for some key definitions that will be used throughout the session:

Frequency (f)

- This is the number of cycles per second measured in hertz (Hz)

- 1 Hz = 1 cycle per second

Amplitude (A)

- The height of a cycle

Phase (Φ)

- The relative value of a variable at a point in a cycle

Wavelength (λ)

- The physical distance over which a cycle repeats

# 3 of 14 – Relationships

The wave speed (c) of electromagnetic (EM) radiation is 300,000 km/s (the ‘speed of light’).

So, as the wave speed (c) is constant:

- Lower frequencies always have longer wavelengths

- Higher frequencies always have shorter wavelengths

Wave speed (c), frequency (f) and wavelength (λ) are related and can be represented by a formula triangle.

f=cλ

Where: c = wave speed, f = frequency and λ = wavelength

c=f×λ

Where: c = wave speed, f = frequency and λ = wavelength

λ=cf

Where: c = wave speed, f = frequency and λ = wavelength

# 4 of 14 – The electromagnetic spectrum

Electromagnetic radiation covers a range of frequencies (spectrum) and associated wavelengths, commonly referred to as the electromagnetic spectrum.

Replaced diagram with table:

| Type of wave | Typical source | Approximate wavelength | Typical uses |
| --- | --- | --- | --- |
| Radio: LW | electronic circuits, stars and space | 1 km | communications, radio, TV |
| Radio: MW | electronic circuits, stars and space | 100 m | communications, radio, TV |
| Radio: VHF | electronic circuits, stars and space | 1 m | communications, radio, TV |
| Microwaves | electronic circuits, cool objects | 1cm (10-2 m) | communications satellites, telephony, heating water and food |
| Infra red (IR) | electronic devices, warm objects, sun | 0.1mm (10-4 m) | magic eyes in security lighting, remote control (eg TV) |
| Light | electronic devices (LED), hot objects, sun | 0.001mm (10-6 m) | seeing, photography, worldwide communication using optical fibres |
| Ultraviolet (UV) | gas discharge, very hot objects, UV lamps, sun | 0.00001mm (10-8 m) | sun-tan lamp, making ions, making vitamin D, water purification plants to kill bacteria |
| X-rays | very fast electrons hitting a metal target | 10-10 m | imaging defects in bones, hidden devices |
| Gamma rays(γ) | radioactive nuclei decaying | 10-12 m | medical tracers, killing cancerous cells, sterilisation |

Click here for source.

Note: energy increases as wavelength becomes shorter.

# 5 of 14 – The radio frequency spectrum

The table below shows how the radio frequency spectrum is divided into discrete frequency bands:

| VLF | LF | MF | HF | VHF | UHF | SHF |
| --- | --- | --- | --- | --- | --- | --- |
| 10 kHz | 100 kHz | 1 MHz | 10 MHz | 100 MHz | 1 GHz | 10 GHz |
| Communication with submarines | LW Radio | MW Radio | Contactless payment cards (RFID) | FM Radio DAB Radio Television | Microwave ovens Digital television Mobile phones Wireless LAN Bluetooth GPS | Radio astronomy Radar systems Wireless LAN Satellite TV |

- The International Telecommunication Union – Radiocommunication Sector (ITU-R) and Institute of Electrical and Electronics Engineers (IEEE) have defined radio bands

- Each band can be further split into channels

- Bandwidth refers to the range of frequencies available

# 6 of 14 – Making waves

Antennas - Transmission

- Two metal rods, half wavelength

- AC current passed through them

- Waves of electric field emitted

Diagram: shows wave pattern of a two rod (di-pole) antenna in transmit mode.

Antennas - Reception

- Incoming electric field waves push electrons in the metal rods back and forth

- An alternating current is produced

Diagram: shows wave pattern of a two rod (di-pole) antenna in receive mode.

# 8 of 14 – Communicating information

Communication involves sending information from A to B via a link. This information is known as the intelligence signal. Generally, of a low frequency - Human voice is in the range of 20 to 20,000 Hz.

A 300Hz radio wave would require an antenna 500km tall! Therefore, higher frequency waves with shorter antennas are more practical.

# 9 of 14 – Types of waveforms

Radio waveforms can be split into two types, analogue and digital:

Analogue

- Values change continuously

- Very susceptible to noise interference

Digital

- Discrete values – 0s and 1s

- Much less susceptible to noise

# 10 of 14 – Analogue modulation

Modulation puts intelligence onto a higher frequency carrier wave.

Demodulation is the reverse process

The formula for a sine wave is y = Asin(2πft + Φ)

Where,

A = amplitude, f = frequency, Φ = phase angle

Any of these can be varied in accordance with the intelligence signal

- Amplitude modulation (AM)

- Angular modulation

  - Frequency modulation (FM)

  - Phase modulation (PM)

# 11 of 14 – Digital Modulation

Symbols represent a predetermined, finite, pattern of binary bits

- Two bit, symbols = 00, 01, 10, 11

- Three bit, symbols = 000, 001, 010, 011, 100, 101, 110, 111

- Originates from the Morse Code Key, hence keying

The characteristics of a carrier signal are altered to convey symbols

- Phase Shift Keying uses a finite number of phases per symbol

- Frequency Shift Keying uses a finite number of frequencies per symbol

- Amplitude Shift Keying uses a finite number of amplitudes per symbol

- Quadrature Amplitude Modulation – Combinations of amplitude and phase

Constellation diagrams are used to show the full symbol ‘alphabet’. 8-PSK Each symbol is encoded as a different phase shift of the carrier sine wave: 0°, 45°, 90°, 135°, 180°, 225°, 270°, 315°

# 12 of 14 – Transmitting and receiving

Transducers convert one form of energy to another:

- Light bulbs convert electricity to EM waves in the visible part of the spectrum

- Antennas convert electricity to EM waves in the radio-communication part

Transceivers can transmit and receive signals.

Radio waves propagate through the atmosphere in different ways

- Lower frequency waves are reflected off the ionosphere (skywaves)

- Medium frequency waves are refracted around obstacles (groundwaves)

- Higher frequency waves must have direct line of sight

- Super high frequency waves

# 13 of 14 – Digital multiplexing

Multiple sources of intelligence signals may share a common carrier. This is achieved by multiplexing, of which there are several types:

Space Division Multiplexing

- Multiple antennas, each slightly out of phase, (802.11n, 802.11a/c, MIMO)

Time Division Multiplexing

- Each source is given a slice of time on a single carrier

Code Division Multiplexing

- Each source is given a unique code to encode or decode on the carrier

Orthogonal Frequency Division Multiplexing

- Each source has access to multiple, narrower, sub-carrier waves

- Lower symbol rate, but over parallel carriers

# 14 of 14 – Summary

Well done. You have completed this session on the fundamentals of electronic communication.

In this session we have:

- Introduced the basic concepts of wave theory

- Explored the electromagnetic spectrum